FGIS Training Guide

# sampling safety

General Safety

- Obey all posted warning signs

- Wear close fitting clothes

Gangways and Ladders

- Check before boarding.  If defective, do not use.

- Ladders - Do not use if defective

Dock Areas

- Be alert when walking on deck or wharfs

- Learn where emergency and equipment and locations are:

- Fire extinguishers

- Life Rings

- Emergency Ladders

- Telephones

Barges and Ships

- Do not sample barges at night unless docked with sufficient lighting

- Use caution when walking on ship decks, barge tops, and ship gangways

- Stay clear of mooring ropes

- Do not remain on barges while they are moving

- Be aware of other vessels in the area that may bump into your barge

- Require applicant to roll back hatches and covers and lock them into place

- Do not sample barges alone

Rail Cars

- Before entering railyard:

  - Immediately notify supervisor, yardmaster, switch crew foreman

  - Inquire about switching or unusual activities

  - Do not sample rail cars alone

  - Before Beginning Sampling:

  - Cease all activities on the track where you will you be working

  - Require track to  be locked out or require derail to be installed on both ends of the string of cars

  - Follow other appropriate locally approved precautions

  - Do not probe rail cars at night unless there is adequate lighting